| S.N | Descriptions of goods |  | Bidder's Offer | Remarks |
| --- | --- | --- | --- | --- |
|  | Manufacture: |  | Chonqing Meiqi Industrial Manufacturing Co.Ltd | Ok |
|  | Brand: |  | Muktinath | Ok |
|  | Type/Model: |  | 4L-6.0P | Ok |
|  | Country of Origin: |  | China | Ok |
| 1 | Descriptions | Technical Data |  |  |
| 1.1 | Product Type | Self – propelled crawl type combine harvester minimum of 100 HP | Self- propelled Crawler type | Ok |
| 1.2 | Work Efficiency | 0.5 to 1.5 bigha per hour | 0.5-0.9 hectare per hour | ok |
| 1.3 | Threshing Efficiency | More than 98% |  | ok |
| 1.4 | Fuel tank capacity | 150 L | 130 L | The fuel tank capacity is lesser than proposed technical data but it does not make any significant differences |
| 1.5 | Crop cutting capacity | 5 to 6 kg per second |  |  |
| 1.6 | Machine Dimension | 5-5.8m*2.4- 3.7m*2.5-3m | 5.15m*2.62m*3.03m | Ok |
| 1.7 | Engine Specification | Diesel engine,4 stroke, Inline, Vertical, Liquid cooled, turbo charge, RPM: 2400-2600, fuel consumption: 7-10 L per hour, Hydraulic | Diesel engine,4 stroke, Water Cooling, 2600 Rpm, 102 HP, Hydrostatic transmission with 52cc hydraulic gearbox | Ok |

|  |  | gearbox with minimum 45cc, Hydrostatic transmission |  |  |
| --- | --- | --- | --- | --- |
| 1.8 | Width of track | 500-600mm | 500mm*90mm*56mm | Ok |
| 1.9 | Header | Telescopic Transverse Conveyor Auger, Cutting width: 2100mm, | Horizontal Cutter, Reel type: Eccentric finger, 2200mm cutting width | Ok |
| 1.10 | Grain Tank Capacity | 1.6 cum | 1.7 m3 | Ok |
| 1.11 | Unloading Speed | 20-50 L per second |  |  |
| 1.12 | Grain loss speed | Equal or less than 1.5% |  |  |
| 1.13 | Steering | Hydraulic | hydraulic | Ok |
| 1.14 | Gear | Min. 3 Forward and Reverse | ZKB 85 gearbox | Ok |
| 2 | Other Features |  |  |  |
| 2.1 | Straw Walker | Axial flow threshing system with vibrating sieves and centrifugal fan for separation | Vibrating sieves + centrifugal fan | Ok |
| 2.2 | Residue Management | Straw chopper/ Baler Compulsory | Baler | Ok |
| 2.3 | Cabin | Semi enclosed, Digital display for monitoring as possible | Semi enclosed, Digital display for monitoring as possible | Ok |

| 2.4 | Lighting | Led or halogen for night time operation | Led or halogen for night time operation | Ok |
| --- | --- | --- | --- | --- |
| 2.5 | Crop Adapted | Rice and other multi crop too | Rice and other multi crop too | Ok |
| 3 | Certification |  |  |  |
| 3.1 | Dealer Authorization Certificate, Warranty Certificate, Performance Test Certificate, Test Report Certificate, ISO certificate etc. |  | Attached | Ok |